# Open-Source Vision-Language-Action (VLA) Models

A simple map of the field — and where SmolVLA (the model I deployed on the SO-100 arm) fits

# What a VLA is

A Vision-Language-Action (VLA) model takes camera images plus a text instruction (e.g. "pick up the bottle and place it in the yellow square") and outputs the robot’s motor commands directly. This is the model family I used to make the SO-100 arm perform the pick-and-place task autonomously.

# How the models differ (three simple axes)

Size: From ~450 million to tens of billions of parameters. Smaller = runs on cheap hardware; larger = generally smarter and more general, but needs big GPUs.

How they produce actions: (a) Autoregressive — predict action “tokens” one by one, like text. (b) Diffusion — denoise a whole motion at once. (c) Flow-matching — a fast “action expert” (what SmolVLA and π0 use).

Openness: Some are fully open (weights + code you can download and run). Some are closed (e.g. Google’s), with no downloadable weights.

# Where SmolVLA fits

SmolVLA is the small, efficient, fully-open option. It is about 450M parameters, built into HuggingFace’s LeRobot framework, and designed for exactly the SO-100/SO-101 arm and community data. It is not the most capable model in the field — bigger models like π0, OpenVLA, and MolmoAct generalize better — but it is the one you can actually train and run on a normal laptop GPU, which is why it worked for our setup. In short: SmolVLA trades peak capability for cost, speed, reproducibility, and a native fit to our stack.

# Comparison of the main open-source VLAs

| Model | Size | Action type | Good at | Open? | Link |
| --- | --- | --- | --- | --- | --- |
| SmolVLA (mine) | ~450M | Flow-matching | Cheap to run; native to LeRobot/SO-100; fully open | Open | paper |
| Octo | 27–93M | Diffusion | Lightweight generalist; fast to fine-tune | Open | site |
| π0 (Pi-Zero) | ~3.3B | Flow-matching | SOTA-class dexterity & cross-robot transfer | Open | openpi |
| π0.5 | ~3B | Flow-matching | Open-world generalization to new homes/tasks | Open | blog |
| OpenVLA | 7B | Autoregressive | Widely-used open generalist research baseline | Open | site |
| OpenVLA-OFT | 7B | Parallel/continuous | ~25–50× faster than OpenVLA, higher success | Open | paper |
| RDT-1B | 1.2B | Diffusion | Bimanual (two-arm) manipulation | Open | code |
| GR00T N1.5 | ~3B | Flow (dual-system) | Humanoids; strong Isaac simulation pipeline | Open | weights |
| MolmoAct | 7B | Action reasoning | 3D spatial reasoning; interpretable plans; most open | Open | blog |
| RT-2 | 12–55B | Autoregressive | Defined the VLA paradigm (research only) | Closed | site |
| Gemini Robotics | undisclosed | Proprietary | Frontier dexterity + reasoning (commercial) | Closed | page |
| ACT (baseline) | ~80M | Action chunking | Data-efficient single task; no language model | Open | paper |
| Diffusion Policy (baseline) | ~100–300M | Diffusion | Smooth single-task control; no language model | Open | site |

# SmolVLA — quick pros and cons

Good at: runs on a laptop GPU (even CPU); native to LeRobot and the SO-100; asynchronous inference for higher control rates; fully open and reproducible; learns from just tens of demonstrations (we used 56).

Gives up: peak dexterity and generalization to hard/novel tasks; rich multi-step language reasoning; explicit 3D/spatial reasoning; bimanual or humanoid scale (those are other models’ specialties).

# What realistically runs on my laptop (RTX 3050, 4 GB)

Runs fine: SmolVLA, Octo, ACT, Diffusion Policy — my practical toolbox today.

Needs a bigger GPU: π0, π0.5, GR00T, RDT-1B (1–3B) — possible only with quantization/offload; fine-tuning wants ≥16–24 GB.

Not self-hostable: OpenVLA / MolmoAct (7B) need 24 GB+; RT-2 and Gemini Robotics have no public weights.

# Key sources

SmolVLA paper (arXiv 2506.01844)

SmolVLA blog (HuggingFace)

LeRobot framework (GitHub)

Open X-Embodiment (cross-robot dataset)

VLA overview (Wikipedia)

Note: figures are simplified for comparison and current as of July 2026 — check the linked papers before quoting exact numbers.